MICROSAMPLER AND FCB1010

| Programming the Midisolutions "Event Processor Plus" to control the MicroSampler with the FCB1010 NRPN CC#98 for MSB and CC#99 for LSB |  |  |  |
| --- | --- | --- | --- |
| 1 | Map note-on X 48-63 to CC#99 MSB val. 32 | Set “once” for all the values under it until another NRPC # is needed |  |
| 2 | Map note-on X 48-63 to CC#98 LSB val. 01 |  |  |
| 3 | Map note-on X 48(C2) to CC#6 val 5 | Select Pattern #1 | Note C2 |
| 4 | Map note-on X 49(C#2) to CC#6 val. 10 | Select Pattern #2 | Note C#2 |
| 5 | Map note-on X 50(D2) to CC#6 val. 19 | Select Pattern #3 | Note D2 |
| 6 | Map note-on X 51(D#2) to CC#6 val. 27 | Select Pattern #4 | Note D#2 |
| 7 | Map note-on X 52(E2) to CC#6 val. 36 | Select Pattern #5 | Note E2 |
| 8 | Map note-on X 53(F2) to CC#6 val. 44 | Select Pattern #6 | Note F2 |
| 9 | Map note-on X 54(F#2) to CC#6 val. 51 | Select Pattern #7 | Note F#2 |
| 10 | Map note-on X 55(G2) to CC#6 val. 58 | Select Pattern #8 | Note G2 |
| 11 | Map note-on X 56(G#2) to CC#6 val. 67 | Select Pattern #9 | Note G#2 |
| 12 | Map note-on X 57(A2) to CC#6 val. 75 | Select Pattern #10 | Note A2 |
| 13 | Map note-on X 58(A#2) to CC#6 val. 84 | Select Pattern #11 | Note A#2 |
| 14 | Map note-on X 59(B2) to CC#6 val. 91 | Select Pattern #12 | Note B2 |
| 15 | Map note-on X 60(C3) to CC#6 val. 99 | Select Pattern #13 | Note C3 |
| 16 | Map note-on X 61(C#3) to CC#6 val. 108 | Select Pattern #14 | Note C#3 |
| 17 | Map note-on X 62(D3) to CC#6 val. 116 | Select Pattern #15 | Note D3 |
| 18 | Map note-on X 63(D#3) to CC#6 val. 125 | Select Pattern #16 | Note D#3 |
| 19 | Map note-on X 64(E3) to CC#99 MSB val. 32 | “Record” | Note E3 |
| 20 | Map note-on X 64(E3) to CC#98 LSB val. 02 |  |  |
| 21 | Map note-on X 64(E3) to CC#6 val. 127 |  |  |
| 22 | Map note-on X 65(F3) to CC#99 MSB val. 32 | “Sampling” | Note F3 |
| 23 | Map note-on X 65(F3) to CC#98 LSB val. 17 |  |  |
| 24 | Map note-on X 65(F3) to CC#6 val. 127 |  |  |
| 25 | Map note-on X 66(F#3) to CC#99 MSB val. 32 | “Audio in” | Note F#3 |
| 26 | Map note-on X 66(F#3) to CC#98 LSB val.18 |  |  |
| 27 | Map note-on X 66(F#3) to CC#6 val. 50 |  |  |
| 28 | Map note-on X 67(G3) to CC#6 val. 120, End. | “Resample” | Note G3 |
|  | Make sure to select“do not process subsequent settings” |  |  |
| You need to program the FCB1010 switches to send the note for each function. “Patterns 1-16” (notes C2 to D#3) “Record” (note E3) “Sampling” (note F4) “Audio in” (noteF#3) “Resample” (note G3) |  |  |  |